MY REPORT STRUCTURE

Abdirahman Farah

[Student iD] 24796328

2025

1.	CPU Simulation	1

1.1	Calculating the sum of values	1

1.2	Completing the stop instruction	2

1.3	Bitwise Operations	3

1.4	Amending the ALU	4

1.5	Output Device	7

1.6	Branch instruction	9

1.7	Loop	12

1.8	Game	13

2.	RISC-V Assembly Language Programming	14

2.1	Test the existing program with minor adjustments	14

2.2	Improve the messages that the program outputs	14

2.3	Enhancing the functionality of the program	16

2.4	Adding a subroutine	17

2.5	Swapping Numbers	18

2.6	Counting down with a loop	21

2.7	Making use of arrays	21

2.8	Designing a game	22

3.	References	23

4.	Appendix A: The CPU Simulation Code	24

5.	Appendix B: The RISC-V Assembly Language Code	26

5.1	Tasks 2.1 to 2.3	26

5.2	Task 2.4 Including a subroutine	27

5.3	Task 2.5 Swapping numbers	29

5.4	Task 2.6 Loop	31

5.5	Task 2.7 Array	31

5.6	Task 2.8 Creating a console based game	31

6.	Appendix C: Use of Generative AI	32

# CPU Simulation

## Calculating the sum of values

### Test data

The two test numbers are 63 and 28. In hexadecimal 63 is 3F and 28 is 1C.

### Test program

v2.0 raw

103F 3000 	# 1. put first value to ACC and address 00

101C 3001        # 2. put second value to ACC and address 01

2100		# 3. add ACC value to value from address 00

3002 	 	# 4. store ACC value at address 02

f000		# 5. stop

### Expected result

The sum is 5B in hexadecimal and 91 in decimal.

### Circuit with program loaded

### Circuit at end of program

### Result

The first three data-memory addresses now contain the two initial values and their total.

## Completing the stop instruction

### Circuit at start of Test

### Circuit at end of Test

### Explanation

The ACC, PC and Data Memory have a clock signal because they need to hold values and only change at the right moment each cycle, so the CPU stays in sync.

## Bitwise Operations

### Test program

v2.0 raw

103F 3000  	# 1. put first value to ACC and store to 00

101C 3001  	# 2. put second value to ACC and store to 01

2100	   	# 3. add ACC value to value from address 00

3002 	 	# 4. store ACC value to address 02

2000 2201  	# 5. load from 00 to ACC, bitwise NAND with value from 01

3003 	 	# 6. store ACC value to address 03

2000 2301  	# 5. load from 00 to ACC, bitwise NAND with value from 01

3004 	 	# 6. store ACC value to address 04

f000

### Expected result

|  | binary | Hex |
| --- | --- | --- |
| First number | 0011 1111 | 3F |
| Second number | 0001 1100 | 1C |
| Bitwise AND of original numbers | 0001 1100 | 1C |
| Bitwise NOT of previous result ( giving NAND) | 1110 0011 | E3 |
| After bitwise OR of original numbers | 0011 1111 | 3F |

### Circuit at end of program

### Result

In the data memory I now see an addition of E3 and also 3F showing now when before it stopped at 5B.

### Explanation

OR sets each output bit to 1 if either input bit is 1, while NAND does AND then flips the result; the instruction decoder selects which ALU operation to perform.

## Amending the ALU

### ALU circuit

### Test Program

v2.0 raw

103F 3000  	# 1. put first value to ACC and store to 00

101C 3001  	# 2. put second value to ACC and store to 01

2100	   	# 3. add ACC value to value from address 00

3002 	 	# 4. store ACC value to address 02

2000 2201  	# 5. load from 00 to ACC, bitwise NAND with value from 01

3003 	 	# 6. store ACC value to address 03

2000 2301  	# 5. load from 00 to ACC, bitwise NAND with value from 01

3004 	 	# 6. store ACC value to address 03

2000 2400  	# 5. load from 00 to ACC, binary shift to the right

3005 	 	# 6. store ACC value to address 05

2000 2501  	# 5. load from 00 to ACC, compare with value from 01

3006 	# 6. store ACC value to address 06

f000

### Expected result

| Test description | Value at 00 | Value at 01 | Right shift of first value | Set greater than value |
| --- | --- | --- | --- | --- |
| First number bigger | 3F | 1C | 03 | 01 |
| First number smaller | 1C | 3F | 01 | 00 |
| Both numbers the same | 3F | 3F | 03 | 00 |

### Circuit at end of program

### Result

In my test the first value is 3F and the second value is 1C, I’m comparing either to see which is larger and which is not resulting in the first value if larger to shift right. If not it will result in it outputting 00 as it wont shift since it isn’t greater. I predicted that if 3F is greater than 1C, it will shift right to 03 and then output 01, if 1C is being compared to 3F it will shift right to 01 and output 00 because it isn’t greater etc.

### Explanation

I used a comparator to check whether the ACC value was greater than the second input. The comparator only gives a 1-bit output, but the rest of my ALU uses 8-bit signals, so I converted it to 8 bits. I did this by keeping the comparator output as the least significant bit and setting the other seven bits to 0. This means the ALU outputs 0x01 when the comparison is true and 0x00 when it is false.

## Output Device

### Test Data and Expected Result

v2.0 raw

48 69 20 41 62 64 69 72 61 68 6D 61 20 33 46

Hi Abdirhma 63

### Test Program

v2.0 raw

2000 5000

2001 5000

2002 5000

2003 5000

2004 5000

2005 5000

2006 5000

2007 5000

2008 5000

2009 5000

200a 5000

200b 5000

200c 5000

200d 5000

f000

### Circuit at end of program

### Result

In this image it shows my TTY printing out what I predicted it would say, Hi Abdirhma 3F.

### Explanation

ASCII is limited to 7 bits, meaning it can only represent 128 characters (IETF, 1969). This makes it unsuitable for non-English characters and symbols. To address this, extended ASCII uses 8 bits to add more characters, but it is not universally standardised, which causes compatibility issues (TechTarget, 2025). Unicode, especially UTF-8, solves this by supporting all ASCII characters and adding many more, allowing for characters from all writing systems (Mozilla, n.d.).

The research helped me understand why ASCII is limited and why Unicode (UTF-8) is now the standard, offering broader character support and better compatibility (IETF, 1969; TechTarget, 2025; Mozilla, n.d.).

## Branch instruction

### ALU circuit and testing

### Test program

v2.0 raw

103F 3000  	# put first value to ACC and store to 00

101C 3001  	# put second value to ACC and store to 01

2000        # load value from address 00 into the ACC

2501 400f   # set ACC value to 1 if data in address 0 is greater than the data in address 01

2000 3002   # load value from address 00 to ACC and store in address 02

2001 3000   # load value from address 01 to ACC and store in address 00

1000 3002   #set the value at address 02 in data memory to be 00

f000        # end program

### Expected Results

I am expecting the program to branch to f000 if address 00 is greater than address 01 after that the program will end. if that happens it will store 3F in address 00. If address 00 is not greater than address 01, the program will continue and store address 00 value into 02, then it will load value from address 01 and store it into address 00 then set the value of address 02 in data memory 00. Then finally end program.

### Results

This is an image of the program running to test if 3F which is stored in address 00 is greater than 01 which then will lead to it branching to f000 which ends the program.

This is the second and third test. The second test was when I switched the two data with address 00 being 1C and address 01 being 3F, this resulting in my program continuing to run and then ended up stopping when it got to both being 3F as they are the same. The final test resulted in the program stopping because it wasn’t smaller or larger.

### Explanation

For my branch instruction, I needed it to jump straight to the stop instruction (f000). I wrote my full program first, then counted the instruction addresses starting from 00 until I got to f000. In my program, f000 is at address 0F, so the branch instruction had to be 400F (4 = branch, 0 = no ALU op, 0F = target address). When the CPU runs the branch, it only jumps if BNEZ = 1 and ACC_NE_Zero = 1, so it loads the PC with 0F instead of doing PC+1.

From research, another common branch is BEQZ (branch if equal to zero). Shakti+1 That one would branch when the accumulator is zero. To add that to my CPU, I’d have a BEQZ output from the instruction decoder, then use a NOT on ACC_NE_Zero (so it becomes “ACC is zero”), and AND them together: BEQZ AND (NOT ACC_NE_Zero). That signal would drive the same PC-input multiplexer to select Addr_or_Imm.

## Loop

### Displaying Hex Output

### Running Loop

### Explanation

## Game

# RISC-V Assembly Language Programming

## Test the existing program with minor adjustments

### Starter Program

### Expected Result

The total should be 91

### Program code

### Result

The total is 91 as expected

## Improve the messages that the program outputs

### Program Code

This is the code for the total message

This is the lines of code following to show the total being called

### Results

### Explanation

Clear messages help the user interact and understand what to do and also how to use the programs functionality.

## Enhancing the functionality of the program

### Program Code

### Expected Result

| First number | 63 | 3F |
| --- | --- | --- |
| Second number | 28 | 1C |
| Addition of original numbers | 91 | 5B |
| Bitwise OR of original numbers | 63 | 3F |
| Bitwise AND of original numbers (intermediate for NAND) | 28 | 1C |
| Bitwise NOT of previous result (giving NAND) | -29 | FFFFFFE3 |

### Results

The results lined up with my prediction as shown In the table and Run I/O with all the values matching.

### Explanation

First I started by doing the OR function which was or s3,s0,s1, after that I began the nand operation but the nand doesn’t have a one step function so I split it into 2. Firstly I did and function which was and t0, s0, s1. Which stored it into t0, then I did not s4, t0 which flipped it and turned it into a nand gate finally.

### Comparison

Logisim (CPU simulation): I wrote raw hex machine code (v2.0 raw). The program stores inputs in Data Memory [00] and [01], then uses opcodes to do ADD (2100 → store to [02]), NAND (2000 2201 → store to [03]) and OR (2000 2301 → store to [04]). I checked results by comparing the Data Memory hex values to my expected results (5B, E3, 3F).

RARS (RISC-V): I wrote assembly (add, or, and NAND as and then not) and checked correctness using Run I/O and register values. NAND prints as -29 because RARS uses 32-bit signed 2’s complement, so E3 becomes 0xFFFFFFE3. Value of research confirmed why NAND can show as a negative number in RARS even though the bit pattern matches the CPU simulation.

## Adding a subroutine

### Subroutine code

### Result

### Explanation

Using subroutines in programming/risc v allows the user to break down long lines of code into more readable and neat lines of code which makes it easier to understand and follow through for them and also anyone who uses the code after them. It also allows them to debug it a lot easier in the long run. Jal stands for jump and link, Ret stands for return. I used jal to call the subroutine and jump to it, I used ret to end the subroutine and return back to the line underneath which here would be line 63.

## Swapping Numbers

### Program Code

### Expected Result

| Test description | Value at s0 | Value at s1 |  |
| --- | --- | --- | --- |
| First number bigger | 63 | 28 | 63, 28 |
| First number smaller | 28 | 63 | 63, 28 |
| Both numbers the same | 28 | 28 | 28 28 |

### Result

This was the first test and it was the same as my expected.

This was the second test and it was the same my expected.

This was the last test and it was the same as my expected

### Alternative approach

### Explanation

The way I went with doing the swap code was by using the commands sgt and bnez which both stand for set greater than and branch if not equal to zero. In the alternative approach I used slt and beq instead which stands for branch if equal and set less than. I found that they were both the same difficulty and skill one was just dealing with it by larger than and the other was dealing with it the opposite way. They were pretty much the same similar way of code but they were different because slt is often a native instruction and sgt is often a pseudocode instruction.

### Comparison

In my Logisim CPU, swapping meant moving values between Data Memory addresses using load/store steps and a temporary address so I didn’t overwrite anything. In RISC-V, the swap is simpler because the values are already in registers, so I just copy register-to-register. This can be shown throughout this task such as add  s2, s0, zero, add  s0, s1, zero, add  s1, s2, zero. However in the cpu simulation you are using the command 3 to store a value to the acc. E.g 2000 2201, load from 00 to ACC, bitwise NAND with value from 01. This was what I understood from my experience with the work done and I couldn’t find references to research from.

## Counting down with a loop

### Loop Code

### Results

### Alternative approach

### Explanation

## Making use of arrays

### Pseudocode

### Expected Results

### Result

### Explanation

### Comparison

## Designing a game

# References

IETF (1969) RFC 20: ASCII format for network interchange. Available at: https://datatracker.ietf.org/doc/html/rfc20. Accessed: 2 January 2026.

Mozilla (n.d.) UTF-8 - Glossary. MDN Web Docs. Available at: https://developer.mozilla.org/en-US/docs/Glossary/UTF-8. Accessed: 2 January 2026.

TechTarget (2025) ASCII (American Standard Code for Information Interchange). Available at: https://www.techtarget.com/whatis/definition/ASCII-American-Standard-Code-for-Information-Interchange. Accessed: 2 January 2026.

University of Delaware (n.d.) Branch Equal to Zero. Available at: (online). Accessed: 5 January 2026. EECIS
Waterman, A. et al. (2016) The RISC-V Instruction Set Manual, Volume I: User-Level ISA (Version 2.1). University of California, Berkeley. Available at: (online). Accessed: 5 January 2026.

# Appendix A: The CPU Simulation Code

The code within this section needs to be as plain text which could be copied and pasted into another file to test your circuit.

This is the code from task 1.1

v2.0 raw

103F 3000  	# 1. put first value to ACC and store to 00

101C 3001  	# 2. put second value to ACC and store to 01

2100	   	# 3. add ACC value to value from address 00

3002 	 	# 4. store ACC value to address 02

2000 2201  	# 5. load from 00 to ACC, bitwise NAND with value from 01

3003 	 	# 6. store ACC value to address 03

2000 2301  	# 5. load from 00 to ACC, bitwise NAND with value from 01

3004 	 	# 6. store ACC value to address 04

f000

this is the code from task 1.3

v2.0 raw

103F 3000  	# 1. put first value to ACC and store to 00

101C 3001  	# 2. put second value to ACC and store to 01

2100	   	# 3. add ACC value to value from address 00

3002 	 	# 4. store ACC value to address 02

2000 2201  	# 5. load from 00 to ACC, bitwise NAND with value from 01

3003 	 	# 6. store ACC value to address 03

2000 2301  	# 5. load from 00 to ACC, bitwise NAND with value from 01

3004 	 	# 6. store ACC value to address 04

f000

This is the code from task 1.4

v2.0 raw

103F 3000  	# 1. put first value to ACC and store to 00

101C 3001  	# 2. put second value to ACC and store to 01

2100	   	# 3. add ACC value to value from address 00

3002 	 	# 4. store ACC value to address 02

2000 2201  	# 5. load from 00 to ACC, bitwise NAND with value from 01

3003 	 	# 6. store ACC value to address 03

2000 2301  	# 5. load from 00 to ACC, bitwise NAND with value from 01

3004 	 	# 6. store ACC value to address 03

2000 2400  	# 5. load from 00 to ACC, binary shift to the right

3005 	 	# 6. store ACC value to address 05

2000 2501  	# 5. load from 00 to ACC, compare with value from 01

3006 	# 6. store ACC value to address 06

f000

this is the code from task 1.5

v2.0 raw

2000 5000

2001 5000

2002 5000

2003 5000

2004 5000

2005 5000

2006 5000

2007 5000

2008 5000

2009 5000

200a 5000

200b 5000

200c 5000

200d 5000

f000

this is the code from task 1.6

v2.0 raw

103F 3000  	# put first value to ACC and store to 00

101C 3001  	# put second value to ACC and store to 01

2000        # load value from address 00 into the ACC

2501 400f   # set ACC value to 1 if data in address 0 is greater than the data in address 01

2000 3002   # load value from address 00 to ACC and store in address 02

2001 3000   # load value from address 01 to ACC and store in address 00

1000 3002   #set the value at address 02 in data memory to be 00

f000        # end program

# Appendix B: The RISC-V Assembly Language Code

The code within this section needs to be as plain text which could be copied and pasted into RARS to test your work. You may include more subheadings.

## Tasks 2.1 to 2.3

.data

enterMsg1: .string "Please use the last 4 digits of your student id as two 2-digit numbers \n"

enterMsg2: .string "Please enter your first set of digits now \n"

enterMsg3: .string "Enter your next number now \n"

totalMsg: .string "The total is "

OrMsg :		.string "or value is "

NotMsg: 	.string " Not value is "

AndMsg: 	.string "And value is "

.text

###

# output the first instruction text to the console

addi a7, zero, 4

la a0, enterMsg1

ecall

# output the second instruction text to the console

addi a7, zero, 4

la a0, enterMsg2

ecall

# read an integer from keyboard input and store in s0

addi a7, zero, 5

ecall

add s0, zero, a0

# output the text asking for the next number to the console

# then receive the input and store in s1

addi a7, zero, 4

la a0, enterMsg3

ecall

addi a7, zero, 5

ecall

add s1, zero, a0

## add the two values together and store in s2

add s2, s0, s1

or s3, s0, s1

and t0, s0, s1

not s4, t0

##output the total

addi a7, zero, 4

la a0, totalMsg

ecall

# output the value from s2

add a0, s2, zero

addi a7, zero, 1

ecall

# Output message and value for bitwise OR

addi a7, zero, 4

la a0, OrMsg

ecall

# output the value from s3

add a0, s3, zero

addi a7, zero, 1

ecall

# Output message and value for bitwise NAND

addi a7, zero, 4

la a0, NotMsg

ecall

# output the value from s4

add a0, s4, zero

addi a7, zero, 1

ecall

addi a7, zero, 10

ecall

## Task 2.4 Including a subroutine

.data

enterMsg1: .string "Please use the last 4 digits of your student id as two 2-digit numbers \n"

enterMsg2: .string "Please enter your first set of digits now \n"

enterMsg3: .string "Enter your next number now \n"

totalMsg: .string "The total is "

OrMsg :		.string "\nor value is "

NandMsg: 	.string "\nNand value is "

.text

###

# output the first instruction text to the console

addi a7, zero, 4

la a0, enterMsg1

ecall

# output the second instruction text to the console

addi a7, zero, 4

la a0, enterMsg2

ecall

# read an integer from keyboard input and store in s0

addi a7, zero, 5

ecall

add s0, zero, a0

# output the text asking for the next number to the console

# then receive the input and store in s1

addi a7, zero, 4

la a0, enterMsg3

ecall

addi a7, zero, 5

ecall

add s1, zero, a0

## add the two values together and store in s2

add s2, s0, s1

or s3, s0, s1

and t0, s0, s1

not s4, t0

##output the total

la a0, totalMsg

add a1, zero, s2

jal outputTextSubroutine

# Output message and value for bitwise OR

la a0, OrMsg

add a1, zero, s3

jal outputTextSubroutine

# Output message and value for bitwise NAND

la a0, NandMsg

add a1, zero, s4

jal outputTextSubroutine

#exit program

addi a7, zero, 10

ecall

#subroutine

outputTextSubroutine:

# Output message

addi a7, zero, 4

ecall

# output the value

add a0, a1, zero

addi a7, zero, 1

ecall

ret

## Task 2.5 Swapping numbers

.data

msg1: .string "Enter first number:\n"

msg2: .string "Enter second number:\n"

outmessage: .string "Sorted high to low: "

nl: .string "\n"

.text

.globl main

main:

# read s0

addi a7, zero, 4

la   a0, msg1

ecall

addi a7, zero, 5

ecall

add  s0, a0, zero

# read s1

addi a7, zero, 4

la   a0, msg2

ecall

addi a7, zero, 5

ecall

add  s1, a0, zero

# swap-if-needed

sgt  t1, s0, s1

bnez   t1, sorted    # if already sorted, skip swap

add  s2, s0, zero

add  s0, s1, zero

add  s1, s2, zero

sorted:

# output message

addi a7, zero, 4

la   a0, outmessage

ecall

# print s0

addi a7, zero, 1

add  a0, s0, zero

ecall

# print separator as single character (space)

addi a7, zero, 11

addi a0, zero, 32

ecall

# print s1

addi a7, zero, 1

add  a0, s1, zero

ecall

# newline

addi a7, zero, 4

la   a0, nl

ecall

# exit

addi a7, zero, 10

ecall

### Alternative approach

.data

msg1: .string "Enter first number:\n"

msg2: .string "Enter second number:\n"

outmessage: .string "Sorted high to low: "

nl: .string "\n"

.text

.globl main

main:

# read s0

addi a7, zero, 4

la   a0, msg1

ecall

addi a7, zero, 5

ecall

add  s0, a0, zero

# read s1

addi a7, zero, 4

la   a0, msg2

ecall

addi a7, zero, 5

ecall

add  s1, a0, zero

# swap-if-needed

slt  t1, s0, s1

beq  t1, zero, sorted    # if already sorted, skip swap

add  s2, s0, zero

add  s0, s1, zero

add  s1, s2, zero

sorted:

# output message

addi a7, zero, 4

la   a0, outmessage

ecall

# print s0

addi a7, zero, 1

add  a0, s0, zero

ecall

# print separator as single character (space)

addi a7, zero, 11

addi a0, zero, 32

ecall

# print s1

addi a7, zero, 1

add  a0, s1, zero

ecall

# newline

addi a7, zero, 4

la   a0, nl

ecall

# exit

addi a7, zero, 10

ecall

## Task 2.6 Loop

## Task 2.7 Array

## Task 2.8 Creating a console based game

### Stage 1

### Stage 2

### Stage 3

# Appendix C: Use of Generative AI

In this section, name the tool used and paste the full chat prompts and outputs used towards responses for this assessment.